Airspace Brief Outline

1.  4 Types of Airspace: Controlled, Uncontrolled, Special Use, and Other

A.  First, let’s talk about controlled airspace ( AIM chap 3 )

|  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- |
| Dimensions | 18,000 MSL up to and including FL 600, | surface to 10,000 feet MSL individually tailored Like upside-down wedding cake | Surface to 4,000 agl (but charted in msl ) w/in 5 mile radius. 1200 AGL to 4000 agl w/in 10 NM. | Surface to 2500’ agl with a 4 mile ring around airports w/ a control tower | If the airspace is not Class A, B, C, or D, and it is controlled airspace, it is Class E airspace. |
| Requirements to enter | 1 Radio, 2 Nav Equip 3Transponder 4Clearance 5Instrument license | 1Clearance 2Certificate (private pilot or getting one) 3Radio 4Transponder 5Nav equip if IFR | 1Transponder, 2Radio 3Est. Comm. | 1Radio 2Est. Comm. | None |
| VFR Cloud Clearances | None | 3 miles vis, clear of clouds | 500’ below clouds 1000’ above 2000’ horizontal 3 miles vis | 500’ below clouds 1000’ above 2000’ horizontal 3 miles vis | 1. Less than 10,000 msl: 500’ below clouds 1000’ above 2000’ horizontal 3 miles vis 2. Above 10,000 msl 1,000’ below clouds 1,000’ above 1 mile horizontal, 5 mile vis |
| Depiction on VFR Sectional | Not depicted | Thick blue line | Solid Magenta Lines | Dashed blue line | See below |
| Depiction on IFR low chart | Not depicted | Thin blue line | Segmented blue line | Not depicted, box w/ a D inside it | Federal airways are depicted |

i. Seven Types of Echo, remembered by the acronym   “SET FOOD”

| Type | Description / Depiction on VFR sectional |
| --- | --- |
| Surface | Surface area around certain class E airport. Contains all instrument approach procedures. / Segmented magenta line |
| Extension | Extensions of B, C, and D surface airspace that contain instrument approach procedures without imposing the class B,C, or D comm. requirements on those pilots operating VFR. / Segmented magenta lines |
| Transition | Either 700’(faded magenta) or 1200’ (faded blue) used to transition from the terminal to the en-route environment |
| Federal Airways | Unless otherwise specified: From 1200’ agl up to but not including FL 180. shown by light blue lines on VFR charts |
| Offshore | Provides controlled airspace beyond 12 miles off the coast of the US. |
| Other | Everything else that is Echo, also includes the airspace above FL 600 |
| Domestic En Route | Provides controlled airspace to those areas where there is a requirement for ATC to provide service, but the federal airway system is not adequate |

B.  Next, we have Uncontrolled:

|  | Dimensions | Requ to enter | Depiction |
| --- | --- | --- | --- |
| Golf | None | None | None, except on charts out west where class Golf goes all the way from14,500 msl to the surface unless shown otherwise |

Golf Cloud Clearance

|  | Less than 1200’ agl | Greater than 1200’ agl but less than 10,000 msl | Greater than 1200’ agl and also greater than 10000’ msl |
| --- | --- | --- | --- |
| Day | 1 mile vis and clear of clouds | 500’ below clouds, 1000’ above, 2000 feet horizontal, 1mile vis | 1,000’ below clouds 1,000’ above 1 mile horizontal, 5 mile vis |
| Night | 500’ below clouds 1000’ above 2000’ horizontal 3 miles vis | 500’ below clouds 1000’ above 2000’ horizontal 3 miles vis | Same as above |

But, concerning VFR weather mins, OPNAV (5.2.4) says:   outside of controlled airspace, helicopters may be operated below 1200’ agl clear of clouds when the visibility is less than 1 mile if operated at a speed that allows the pilot adequate opportunity to see and avoid other traffic and obstacles.

Concerning Transponders, AIM ( 4-1-19) says:

1. we need one at or above 10,000 msl except in airspace that is below 2,500 AGL  like the mountains.

2. within or across the ADIZ

3. Within 30 NM of class B below 10,000.

4. Within and above all class C up to 10,000 MSL

5. Within 10 miles of certain airports, except the airspace that is outside Class D and below 1200’ AGL.

Special VFR:   ( get them from the RWOP chap 6 ).  Min Wx at South Whiting is ceiling of 500’ agl and 1 mile vis.  But, the CO can authorize special VFR below 500’ and 1 mile vis.  At south whiting you need searchlight on, landing light on, position lights on steady bright.  Read more in RWOP about it.

Draw Chart:   from unlimited down to the deck, East and West USA

C.  Third, there is Special Use Airspace, remembered by the acronym  “MCPRAW” ( AIM chap 3)

| Type | Regulatory? | Why exist | Entry Req | Where Charted | Find info about it | Additional details |
| --- | --- | --- | --- | --- | --- | --- |
| MOA | Non-Reg (means compliance not mandatory by law | High volume of military Opps exist | No specific requirements(VFR aircraft should be extremely vigilant | Low charts, VFR sectionals, terminal area charts | Chart Tabs, AP | Contact FSS within 100 nm for status |
| Controlled Firing Area | Non – Regulatory | For controlled fires such as Rifle range at Camp Pendleton | No specific Requirement | Not charted b/c it does not pose a threat to non-participating aircraft | You can’t | An observer will call a cease fire on the range if a non participating aircraft wanders into the area |
| Prohibited Area | Regulatory ( means mandatory compliance by law | To protect areas of national welfare or security ( white house ) | Prohibited (special approval required like Air Force 1 ) | Low charts, VFR sectionals, terminal area charts, approach plates | Chart Tabs, AP | None |
| Restricted Area | Regulatory | Hazard to non-participating aircraft ( often invisible like missile fire) | Must be cleared by controlling agency or have prior approval | Low charts, VFR sectionals, terminal area charts, approach plates | Chart Tabs, AP | Joint Use (civilian control agency takes responsibility for the RA). Non-joint use (military controls it all the time) |
| Alert Area | Non-Reg | High volume of pilot training (civilian or military) | No specific requirement. | Low charts, VFR sectionals, terminal area charts | Chart Tabs, AP | None |
| Warning Area | Non-reg | Hazard to non-participating aircraft | None for civilians, military aircraft must contact the controlling agency | Low charts, VFR sectionals, terminal area charts, approach plates | Chart Tabs, AP, Section C of IFR Supp (FACSFAC | (similar to restricted area but outside US airspace) |

D.  Finally, we have Other Airspace, remembered by the acronym  “ MANTVTP “ ( AIM chap 3)

| Type ( AIM chap 3 section 5 ) | Description | Clearance ? |
| --- | --- | --- |
| Military Training Route | IFR routes and VFR routes. Identified by an I or V before the route #. If the # has 3 numbers ( such as VR207 ) have segments above 1500’ agl. Routes with 4 # identifiers (such as IR 1206) means all of the route is located below 1500’agl. | Non Regulatory. But, not participating aircraft should use extreme vigilance. |
| Airport Advisory Areas | Located w/in 10nm of an airport w/out a control tower but where an FSS is located on the airport. At such locations the FSS provides complete local advisory services to arriving and departing traffic. | Non Regulatory, but Pilots are encouraged to participate in the advisories. |
| National Security Areas | Established at areas where we need increased security of ground facilities. May become temporarily prohibited | Normally non-reg. Regulatory prohibitions disseminated through NOTAMS. Unless prohibited, pilots requested to voluntarily avoid flying through NSA. |
| Terminal Radar Service Area | As military we treat is just as a class C airspace | Non-regulatory, but for military, compliance is mandatory |
| VFR flyways and VFR corridors | Corridors are a hole through class B, you may operate w/out a clearance or communication with ATC. VRF flyways are for planning purposes. An ATC clearance is required | Corridors are non reg, but for the flyways you must have a clearance. |
| Temporary Flight Restrictions | To protect space agency operations, disaster relief operations, the president, etc. Disseminated via NOTAM beginning w/ words “Flight Restriction”. | You need ATC clearance. |
| Parachute Jump Areas | Pilot should make all calls on CTAF before, during, and after the jump | Non regulatory |