## Pre-start Checklist

| - Pedals | ADJUSTED |
| --- | --- |
| - Seatbelt and inertia lock | “Fastened and Adjusted” |
| - Flight controls | FREE AND COUPLED |
| - Searchlight | OFF |
| - Lower circuit breakers | IN |
| - © Lower avionics Master Switch | OFF |
| - B Avionics | OFF (check UHF, VHF and XPDR) |
| - © Voltmeter select switch | MAIN BAT |
| - ENG ANTI-ICING | OFF |
| - HYD SYSTEM | ON |
| - © Normal / Recover Switch | NORMAL / RECOVER |
| - Instruments | “Static Check. All gauges zero. Clock wound, correct time, OAT ___ o C” |
| - Radar altimeter(s) | SET (To Zero) |
| - ECS Switch | OFF |
| - Overhead switches | OFF |
| - Overhead circuit breakers | IN |
| - INST LT | ON / OFF |
| - © Upper Avionics Master Switch | OFF |
| - Cabin heat Valve | OFF (Located in overhead) |
| - Cockpit lights | OFF |
| - © Rotor brake lever | Stowed |
| - ANTI COLL LT | ON / OFF |
| - POS LT FLASHING | ON / OFF |
| - Helmets | ON |
| - Battery / APU | ON, “Battery __ v (24v min) |
| - ICS © © © | CHECK - NORMAL / EMERG switch NORMAL - INT VOL knobs – AS DESIRED c) Headphone audio select buttons – AS DESIRED |
| - AUD / MUTE Switch | AUD THEN MUTE Intermittent Engine out horn <55% ±3 Ng Steady Rotor Low <90%Nr |
| - FIRE DET TEST | TEST (<15 sec to avoid damage to sys) |
| - CAUTION LT TEST T-xmsn oil press H-hyd press E-eng out R-rotor rpm low G-gen fail © S-stby gen | “Five/Six Normal, full panel, continuity checks.” (Hold button in for 2 seconds – check for continuity of all chip light circuits plus clear chip light) |
| - FORCE TRIM | ON |
| - FUEL quantity | CHECK, “indicates ____ gal.” |

Start Checklist

| - Twist grip | CLOSED (Check fuel open, idle stop, and close positions first) |
| --- | --- |
| - Fuel valve | ON, “Fuel pressure in the green.” (4-30) |
| - Rotors | “Clear on right, no tie downs” |
| - Fire guard | SET |
| - ENGINE START | “Sir, you get the fuel valve in case of a Hot Start. Tell me if the rotors aren’t turning by 25% Ng, Starting on the____ ” Starter switch ON ____ Volts (>17v) © Battery Relay Caution Light (on / off) Rise in Eng Oil Pressure TOT less than 150o C 15% Ng. Twist grip to FLIGHT IDLE Light Off – monitoring TOT Transmission pressure rising (Must be positive by 30%Nr) Ng at or above 58%, Starter Off © Battery Relay Light – Checked Off |
| - ENG / XMSN oil pressures | “Engine Oil Pressure in the green (Secure if <50 PSI, up to 170 ok if cold) transmission oil pressure normal for this phase.” (Must be pos, 50 psi ok if cold) |
| - POSITION LIGHTS | STEADY ON / OFF |
| - APU | REMOVED / NOT USED |
| - BAT switch | ON |
| - FORCE TRIM | ON |
| - AUD / MUTE | AUD |
| - Instruments | CHECK |
| - Caution Panel | CHECKED NORMAL (Gen Fail and Rotor Low RPM) (xmsn oil press light may still be on, takes a while for press to build) |

Pre-Takeoff Checklist

| - Twist grip | 70% Ng Do not accel above flt Idle until Oil temp >15o, Press is <130 and xmsn press <50 |
| --- | --- |
| - © Voltmeter select switch | MAIN GEN |
| - MAIN GEN switch | RESET the ON “Generator light out, rise in volts and Load” |
| - © STBY GEN switch | ON, LIGHTS OUT |
| - © STBY ATT IND switch | ON, GYRO CAGING |
| - © Avionics Master switches | ON |
| - Avionics | ON (BOTH (UHF) ON (VHF) AND STBY(XPDR)) |
| - Loadmeter | BELOW 50% |
| - ECS | ON / OFF / FAN. Do not energize if Load >50% |
| - Flight Controls | ## “Aircraft Clear Around” |
| # WARNING When performing the checks below; at no time shall the controls be left unattended |  |
| Cyclic FT button – DEPRESS |  |
| Check for cyclic freedom of movement |  |
| Cyclic FT button – RELEASE |  |
| Move the cyclic in an “X” pattern checking tip-path plane |  |
| # WARNING Do not displace the cyclic so much as to contact the static stops. |  |
| Yaw Check |  |
| © Apply pressure to both pedals and displace left pedal approximately 2 inches forward, then release. Attempt to neutralize pedals by pulling on left pedal only, then release. Note pedals return to trimmed. |  |
| FORCE TRIMM – OFF |  |
| Cyclic- X-pattern Pedals-3 inch depress Collective check – raise 4” and note Low Nr light and horn. Full down |  |
| Hydraulic Check |  |
| HYD SYSTEM switch – OFF |  |
| Note HYDRAULIC PRESSURE caution light illuminates. Check cyclic (using “X” pattern) and collective for freedom of movement. |  |
| HYD SYSTEM switch – ON |  |
| Note HYDRAULIC PRESSURE caution light goes out. |  |
| Force Trim – ON |  |
| CONTROL CHECK COMPLETE |  |

| - © Servo indicators | CENTERED |
| --- | --- |
| - © AFCS controller test | Test button Press Test 1 light AMBER STAB light Green FT light Green ALT light Green ALT Warn light Amber FCS caution light On Test button Press Test 2 light Amber STAB button Press and Check Green ALT button - Press and check Green. Check light out and blinking amber on after ≈ 0.5 sec. And out within 10-12 sec. Test Button Press – Check test number no longer illuminated Cyclic FCS button – Check that both cyclics capable of engaging / disengaging AFCS STAB. ### STAB OFF |
| - ENG ANTI-ICING | If OAT >10oC “NOT REQ. CHECKED OFF” “Anti-Ice on, rise in TOT Anti-Ice off, Decrease in TOT.” |
| - Twist Grip | Gauges and Caution Lights Normal Signal to Lineman, Twist Grip Full Open Not to exceed 40% torque” |
| - Nf / Nr | 100% |
| - Flight Idle (From the left Seat) | Ng stabilized at __ % (62% +/- 3%) |
| - © Volt meter select switch | MAIN BAT ± 0.5Vdc of Main Gen 115 Vac ± 4 MAIN GEN 26.5 to 29.0 Vdc 115 Vac ± 4 STBY GEN ± 0.5Vdc of Main Gen 115 Vac ± 4 STBY BAT 24 to 26 Vdc 115 Vac ± 4 ESS1 -1 ± .5Vdc of Main Gen 115 Vac ± 4 ESS2 -1 ± .5Vdc of Main Gen 115 Vac ± 4 NON ESS ± .5Vdc of Main Gen 115 Vac ± 4 FCS INV 0 VDC / 115 Vac ± 4 MAIN GEN Select |

## COMM / NAV Checklist

CLOCK – “SET AND WOUND”

- Set to Proper Time

- Wind if needed. (Not excessively tight)

- Check second hand for reset / start.

DME TEST

- © Ensure RAD button is out on NAV1. Press DME test button. Check that indicator reads 0.0 or 0.1 “ZERO POINT ZERO/ONE”

ATTITUDE GYRO- “Caged”

- Cage by pulling lower right side knob out. Note: only cage if needed.

BAR ALT – “Set to Field Elevation”

- Set bar alt to field elevation.(178 ft)

RMI – “Slaved and Aligned”

- RMI should be aligned with wet compass in conjunction with the compass correction card plus or minus 10 degrees. NOTE: Ensure the slaving adapter switch is in the slave position, air-condition / fan off, defog blower off, landing / search light off, pitot heat switches off) (SLAPD)

MARKER BEACON “Six Illuminated”

a) © Pushing the sensitivity switch to the test position. This should illuminate all the AOM segment lights.

RADAR ALTIMETER – “Test and Set”

- Set DH bug to 25’

- Press the test button. The height indicator should read 50 +/- 5 and the off flag should be visible.

- Release test button. DH light and tone should be evident.

- Reset DH tone

- Set DH bug as desired

VHF - “Test and Set”

- Push in COMM2 button on the bottom row of the audio control panel.

- Select 2 with MIC selector knob.

- Set 121.95 in the “use” position. (TWR-121.4)

- Pull the on/off volume control switch out. Listen for static and push back in.

- Squeeze the trigger and look for a “T” in the transmitter indicator position.

- Push out the COMM 2 button - Select 1 with MIC elector knob.

B VOR - 112.3 and 115.9 “Tuned and Set”

NAV1 and HSI, CDI– “Test and Set”

a) © Set channel to 70X and push the USE button in

b) © Push in the DME button on the audio control panel and listen for ID

c) © Twist in the course indicated by the #1 needle

d) © Fine tune until the course indicator centers.

e) © Rotate HSI / CDI Course pointer 10o right and left, verifying that the deviation needle swings to full deflection both ways.

f) © Rotate HSI / CDI Course pointer to the radial indicated on RMI needles. Deviation needle should center with “FROM” Indicated.

e) © At NDZ, Check the DME at 1.4 NM ± .5 From NSE on both NAV1 / NAV2

GPS and HIS, CDI– “Test and Set”

a) © Set both sides of cockpit to GPS (NAV2)

b) © Check DME at 34.5 miles, FROM flag, HIS/CDI at ½ scale dev. right.

c) © Check OBS out 315 °, RMI 130 °, and single needle pointing 130 °

d) © Place altimeter setting in KLN-900 and approve. Note Current database and Acknowledge.

TRANSPONDER – “Test and Set”

- Function selector switch turned to test position. The display will illuminate all 8’s and 4 mode indicators(ALT on IDT SBY), FL for flight level, R for radar reply, and all 4 digits select caret.

- Function selector switch turned to standby.

- Emergency button pressed, this should display code 7700 until returned.

- Set desired squawk.

ADF- “Tuned and Set”

- © Tune 326 in USE position.

- © Select the ANT mode on the ADF button.

- © Depress the ADF button on the audio control panel and identify the NAVAID. Push out the ADF button on the audio control panel.

- © Check the NAV 1&2 needles in the ADF position on the RMI and positioned at the 90 / 270

- © Select ADF mode and the NAV needles should smartly move toward the station.

- © Switch the NAV needles back to the VOR position.

- © Select the desired frequency

UHF - “Test and Set”Use “U” Shaped pattern starting with Squelch and ending with BRT /Test leaving the preset on CH 5

## Takeoff Checklist

| - Twist grip | FULL OPEN – Nf / Nr 100% |
| --- | --- |
| - Instruments | WITHIN LIMITS |
| - Caution lights | CHECK |
| - © STAB | ON |
| - Force trim | ON |
| - ECS | ON / OFF / FAN |
| - ENGINE ANTI-ICING | ON / OFF |
| - PITOT HEAT | ON / OFF |
| - Shoulder harnesses | LOCKED |
| - Doors | SECURED |
| - Crew and Passengers | SECURED |
| - © Instrument Takeoff Checklist | AS REQUIRED |

## Landing Checklist

| - Twist grip | FULL OPEN - Nf / Nr 100% |
| --- | --- |
| - Shoulder harness | LOCKED |
| - Crew / Passengers | SECURE |
| - LDG LT / SEARCH LT | ON / OFF |
| - ECS | ON / OFF / FAN |

Shutdown Checklist

| - Twist grip | “Flight Idle started on the ___.” |
| --- | --- |
| - © STAB | OFF |
| - ENGINE ANTI-ICING | OFF |
| - LDG LT / SEARCH LT | OFF |
| - © STBY ATT IND | OFF |
| - © STBY GEN | OFF |
| - PITOT HEAT | OFF |
| - DEFOG blower | OFF |
| - Cabin heat valve | OFF |
| - © Avionics Master switches | OFF |
| - B Avionics | UHF OFF, VHF OFF, XPDR OFF |
| - © Voltmeter select switch | MAIN BAT |
| NOTE: After 2 minutes at FLIGHT IDLE, and TOT stabilized for 30 seconds: |  |
| - ECS | OFF |
| - AUD MUTE SWITCH | MUTE |
| - MAIN GEN | OFF |
| - POS LT | FLASHING / ON / OFF |
| - Twist Grip | CLOSE “Good Nr/Nf split, and the fire is out” |
| # WARNING Do not secure engine with fuel shutoff valve. |  |
| - © ROTOR BRAKE | “Nr between 38 and 30% apply” |
| - OVERHEAD SWITCHES | OFF |
| - BAT | “TOT stable below 400oC and Ng zero, battery off” |