TH57C PREFLIGHT EXTRAS

Fuselage Front

NONE

Cockpit

Before turning battery on, put Voltmeter select switch to MAIN BAT

Once battery is on, STBY ATT IND switch ON

Before turning battery off:

Put Voltmeter select switch to STBY BATT.

Make sure standby attitude indicator off flag disappears

Check STBY BATT ON caution light is illuminated and STBY BATT VOLTAGE is >20

NOTE: if <20, a/c is down for IMC flight.

STBY ATT IND switch OFF

Forward Fuselage Right side

NONE

Hydraulic Access Door Right side

Rotor brake master cylinder filler cap	SECURE

Transmission Access Door Right side

NONE

Engine Compartment Right side

Rotor brake	CONDITION AND SECURITY

Aft Fuselage Right side

NONE

Tailboom Right side

NONE

Tail Rotor

NONE

Tailboom Left side

NONE

Aft Fuselage Left side

Standby battery circuit breaker	IN	(in baggage compartment)

Engine Compartment Left side

Standby generator	CONDITION AND SECURITY

Transmission Access Door Left side

NONE

Hydraulic Access Door Right side

NONE

Upper Fuselage Area

GPS antenna	CONDITION AND SECURITY

Rotor System

NONE

Forward Fuselage Left Side

NONE